AXI4 VIP

Implementation Plan

UVM-based Verification IP — Master & Slave Protocol Modules

# 1. Overview

This document describes the implementation plan for a UVM-based AXI4 Full (AMBA 4.0) Verification IP. The VIP covers all five AXI4 channels across two separate protocol modules — master and slave — each with five independent agents. No DUT is used; functional correctness is validated in two testbench modes: back-to-back and loopback.

- AXI4 Full is used (not AXI4-Lite or AXI4-Stream): five independent channels, burst support, unique transaction IDs.

- Separate master and slave protocol modules with no shared code — clean boundary for future DUT insertion.

- One agent per channel; five agents per protocol module (ten total across the testbench).

- W channel is driven only after the corresponding AW transaction completes — enforced by an event in the master virtual sequence.

- All transaction parameters (size, burst type, length) are set at the virtual sequence level and propagate to drivers and monitors through the seq item.

- Two test modes selectable at runtime via plusarg: +TB_MODE=B2B and +TB_MODE=LOOPBACK.

# 2. Directory Structure

All source files live under verif/. The top-level folder holds the two protocol modules and the testbench top. Each of master and slave is self-contained with its own agents, drivers, monitors, interfaces, and sequences.

verif/

├── top/

│   ├── axi4_top.sv              ← clock/reset, interface inst, mode select

│   ├── axi4_master_proto_module.sv

│   └── axi4_slave_proto_module.sv

├── env/

│   ├── axi4_env.sv

│   ├── axi4_scoreboard.sv

│   └── axi4_env_cfg.sv

├── vseq/

│   ├── axi4_master_vseq.sv

│   └── axi4_slave_vseq.sv

├── master/

│   ├── intf/

│   │   ├── axi4_aw_master_intf.sv

│   │   ├── axi4_w_master_intf.sv

│   │   ├── axi4_ar_master_intf.sv

│   │   ├── axi4_b_master_intf.sv

│   │   └── axi4_r_master_intf.sv

│   ├── seq_items/

│   │   ├── axi4_aw_item.sv

│   │   ├── axi4_w_item.sv

│   │   ├── axi4_ar_item.sv

│   │   ├── axi4_b_item.sv

│   │   └── axi4_r_item.sv

│   ├── sequences/

│   │   ├── axi4_aw_seq.sv

│   │   ├── axi4_w_seq.sv

│   │   └── axi4_ar_seq.sv

│   ├── agents/

│   │   ├── axi4_aw_agent.sv

│   │   ├── axi4_w_agent.sv

│   │   ├── axi4_ar_agent.sv

│   │   ├── axi4_b_agent.sv

│   │   └── axi4_r_agent.sv

│   ├── drivers/

│   │   ├── axi4_aw_master_driver.sv

│   │   ├── axi4_w_master_driver.sv

│   │   └── axi4_ar_master_driver.sv

│   └── monitors/

│       ├── axi4_aw_master_monitor.sv

│       ├── axi4_w_master_monitor.sv

│       ├── axi4_ar_master_monitor.sv

│       ├── axi4_b_master_monitor.sv

│       └── axi4_r_master_monitor.sv

├── slave/

│   ├── intf/

│   │   ├── axi4_aw_slave_intf.sv  … (same 5 interfaces, slave clocking blocks)

│   ├── seq_items/       … (same 5 items reused)

│   ├── sequences/

│   │   ├── axi4_b_resp_seq.sv

│   │   └── axi4_r_resp_seq.sv

│   ├── agents/          … (same agent structure)

│   ├── drivers/

│   │   ├── axi4_b_slave_driver.sv

│   │   └── axi4_r_slave_driver.sv

│   └── monitors/

│       └── (passive monitors for AW, W, AR; active for B, R)

├── mem_model/

│   └── axi4_mem_model.sv

└── pkg/

└── axi4_pkg.sv              ← all includes in compile order

# 3. AXI4 Signal Reference

The following tables list all signals on each of the five AXI4 channels, their widths, directions, and the component responsible for driving them in the VIP. Bus widths (addr_width, data_width) are parameterised in axi4_env_cfg and propagated to all interfaces.

## 3.1 Write Address Channel (AW)

| Signal | Width | Direction | Driver | Description |
| --- | --- | --- | --- | --- |
| AWID | 6 | Master → Slave | Master driver | Unique transaction ID; constant across all beats of the burst |
| AWADDR | addr_width | Master → Slave | Master driver | Starting byte address of the first transfer in the burst; must be naturally aligned to AWSIZE |
| AWLEN | 8 | Master → Slave | Master driver | Burst length minus one; number of transfers = AWLEN+1 (0=1 beat, 255=256 beats) |
| AWSIZE | 3 | Master → Slave | Master driver | Transfer size per beat: 2^AWSIZE bytes (000=1B, 001=2B, 010=4B, 011=8B…); must not exceed bus data width |
| AWBURST | 2 | Master → Slave | Master driver | Burst type: 00=FIXED, 01=INCR, 10=WRAP; WRAP requires AWLEN in {1,3,7,15} |
| AWVALID | 1 | Master → Slave | Master driver | Asserted when master has valid address/control; must remain high until AWREADY is seen |
| AWREADY | 1 | Slave → Master | Slave driver | Asserted when slave can accept the AW transfer; handshake completes on posedge clk where both high |

## 3.2 Write Data Channel (W)

| Signal | Width | Direction | Driver | Description |
| --- | --- | --- | --- | --- |
| WDATA | data_width | Master → Slave | Master driver | Write data for the current beat; valid lanes indicated by WSTRB |
| WSTRB | data_width/8 | Master → Slave | Master driver | Byte-lane enables; bit N enables byte lane N of WDATA; derived from AWSIZE and beat address offset |
| WLAST | 1 | Master → Slave | Master driver | Asserted on the final beat of the burst (beat number AWLEN); must match AWLEN exactly |
| WVALID | 1 | Master → Slave | Master driver | Asserted when WDATA, WSTRB, WLAST are valid; must remain high until WREADY seen |
| WREADY | 1 | Slave → Master | Slave driver | Asserted when slave is ready to accept write data for this beat |
| WID | 6 | Master → Slave | Master driver | Write data ID; must match AWID of the associated address beat (AXI3 required; AXI4 optional) |

## 3.3 Write Response Channel (B)

| Signal | Width | Direction | Driver | Description |
| --- | --- | --- | --- | --- |
| BID | 6 | Slave → Master | Slave driver | Matches the AWID of the completed write transaction |
| BRESP | 2 | Slave → Master | Slave driver | Write response: 00=OKAY, 01=EXOKAY, 10=SLVERR, 11=DECERR; issued only after WLAST is accepted |
| BVALID | 1 | Slave → Master | Slave driver | Asserted when BID and BRESP are valid; must remain high until BREADY seen |
| BREADY | 1 | Master → Slave | Master driver | Asserted when master can accept the write response; may be always high |

## 3.4 Read Address Channel (AR)

| Signal | Width | Direction | Driver | Description |
| --- | --- | --- | --- | --- |
| ARID | 6 | Master → Slave | Master driver | Unique read transaction ID; constant across all R-channel beats of this burst |
| ARADDR | addr_width | Master → Slave | Master driver | Starting byte address; must be naturally aligned to ARSIZE |
| ARLEN | 8 | Master → Slave | Master driver | Burst length minus one; slave must return exactly ARLEN+1 R-channel beats |
| ARSIZE | 3 | Master → Slave | Master driver | Transfer size per beat: 2^ARSIZE bytes; must not exceed bus data width |
| ARBURST | 2 | Master → Slave | Master driver | Burst type: 00=FIXED, 01=INCR, 10=WRAP; same rules as AWBURST |
| ARVALID | 1 | Master → Slave | Master driver | Asserted when address/control is valid; must remain high until ARREADY seen |
| ARREADY | 1 | Slave → Master | Slave driver | Asserted when slave accepts the AR transfer; spec requires slave to wait for both ARVALID and ARREADY before asserting RVALID |

## 3.5 Read Data Channel (R)

| Signal | Width | Direction | Driver | Description |
| --- | --- | --- | --- | --- |
| RID | 6 | Slave → Master | Slave driver | Matches the ARID of the corresponding AR transaction |
| RDATA | data_width | Slave → Master | Slave driver | Read data for the current beat; sourced from memory model at the beat address |
| RRESP | 2 | Slave → Master | Slave driver | Read response per beat: 00=OKAY, 01=EXOKAY, 10=SLVERR, 11=DECERR |
| RLAST | 1 | Slave → Master | Slave driver | Asserted on the final beat (beat ARLEN); master uses this to detect burst completion |
| RVALID | 1 | Slave → Master | Slave driver | Asserted when RID, RDATA, RRESP, RLAST are valid; must remain high until RREADY seen |
| RREADY | 1 | Master → Slave | Master driver | Asserted when master can accept read data; may be always high or throttled per beat |

# 4. Environment Config  (axi4_env_cfg)

A single config object is created in the test and pushed into the config DB. Every component retrieves it; no magic numbers appear in any agent or driver.

- addr_width, data_width, id_width — bus widths; strb_width derived as data_width/8.

- tb_mode — enum {B2B, LOOPBACK}; read by tb_top to select interface wiring.

- en_burst_fixed, en_burst_wrap, en_out_of_order — feature flags that gate constraint regions in seq items.

- Per-agent active/passive knobs — override to make any agent passive for a targeted test.

- resp_delay_min, resp_delay_max — back-pressure range for slave B and R drivers.

# 5. Interfaces

Each of the five channels has its own parameterised interface file — one in master/intf/ and one in slave/intf/. Each interface contains two clocking blocks and one inline protocol assertion.

### Clocking blocks

- master_cb — drives VALID and address/control/data signals; samples READY.

- slave_cb  — drives READY (and RDATA/RRESP/BRESP on response channels); samples VALID signals.

- Default skew: 1-step input, 1-step output on posedge clk — prevents driver/monitor race conditions.

### Inline protocol assertions (per interface)

- VALID-stable: once VALID is asserted and READY is low, VALID must not deassert the next cycle (AXI4 spec requirement).

- No X/Z on control signals while VALID is high.

- Slave must not assert RVALID before both ARVALID and ARREADY have been seen (AR channel assertion).

# 6. Sequence Items

Five separate seq item classes — one per channel — live in master/seq_items/ and are reused by the slave side. Using separate items (not a single mega-item) eliminates unused-field constraint interference and keeps each channel driver simple.

### axi4_aw_item / axi4_ar_item

- Fields: id (6b), addr (addr_width), len (8b), size (3b), burst (2b).

- awsize constrained to <= log2(data_width/8) — transfer never exceeds bus width.

- awaddr constrained to natural alignment: addr % (1 << awsize) == 0.

- WRAP burst additionally constrains awlen inside {1, 3, 7, 15} per the AXI4 spec.

### axi4_w_item

- Fields: wdata (data_width), wstrb (data_width/8), wlast (1b), wid (6b).

- wstrb is not randomised freely — it is computed by the driver from awsize and the beat byte offset.

- wlast is not in the item; the driver sets it when beat count reaches awlen.

### axi4_b_item / axi4_r_item

- b_item: bid (6b), bresp (2b). Driven by slave, sampled by master monitor.

- r_item: rid (6b), rdata (data_width), rresp (2b), rlast (1b). Driven beat-by-beat by slave.

# 7. Burst Types

The vseq, drivers, and monitors all interpret AWBURST/ARBURST to compute per-beat addresses and validate boundaries. The table below summarises each type.

| Burst type | AWBURST/ARBURST | Address calculation | Constraints |
| --- | --- | --- | --- |
| FIXED | 2'b00 | Same address for every beat; used for repeated access to a single location (e.g. FIFO) | No constraints on AWLEN or AWADDR alignment beyond AWSIZE alignment |
| INCR | 2'b01 | Address increments by 2^AWSIZE bytes each beat; most common burst type | AWADDR must be aligned to AWSIZE; AWLEN 0-255 all valid |
| WRAP | 2'b10 | INCR within a power-of-two aligned address range; wraps from upper boundary back to lower | AWLEN must be in {1,3,7,15}; AWADDR must be aligned to total burst size (AWSIZE x (AWLEN+1)) |

WSTRB computation — for INCR and WRAP, the active byte lane mask shifts with each beat address. For FIXED, it is the same for every beat. The W driver calls compute_strb(awsize, beat_addr) to generate WSTRB; the W monitor calls the same function to verify received WSTRB.

# 8. Agents, Drivers, and Monitors

Every agent follows the same UVM pattern: sequencer → driver → interface (via clocking block); monitor observes both sides of the handshake and broadcasts a completed item on its analysis port. Active/passive mode is controlled by axi4_env_cfg.

| Channel | Master | Slave | Driver responsibility | Monitor responsibility |
| --- | --- | --- | --- | --- |
| AW | ACTIVE | PASSIVE | Assert AWVALID with address/control; hold until AWREADY; pass AWLEN, AWSIZE, AWBURST to W sequence via event | Sample on AWVALID & AWREADY; broadcast axi4_aw_item to scoreboard and slave vseq FIFO |
| W | ACTIVE | PASSIVE | Drive WDATA, WSTRB (computed from AWSIZE and beat offset), WLAST (on beat AWLEN), WVALID each beat; wait WREADY per beat | Sample each beat on WVALID & WREADY; reconstruct full burst; broadcast axi4_w_item on WLAST |
| AR | ACTIVE | PASSIVE | Assert ARVALID with address/control; hold until ARREADY; pass ARLEN, ARSIZE, ARBURST to slave vseq | Sample on ARVALID & ARREADY; broadcast axi4_ar_item to scoreboard and slave vseq FIFO |
| B | PASSIVE | ACTIVE | Assert BREADY (may be always high or delayed); receive BID and BRESP | Sample on BVALID & BREADY; broadcast axi4_b_item; scoreboard matches BID to pending AWID |
| R | PASSIVE | ACTIVE | Assert RREADY per beat (may be always high or throttled); detect RLAST for burst end | Sample each beat on RVALID & RREADY; reconstruct burst; broadcast axi4_r_item on RLAST |

### W driver — burst-aware beat loop

- Receives awlen, awsize, and awburst from the AW seq item passed via the master vseq event.

- Drives awlen+1 beats; computes WSTRB per beat from awsize and the current beat address.

- Asserts WLAST on the final beat (beat index == awlen); waits WREADY each beat before advancing.

### R driver — burst-aware beat loop

- Receives arlen, arsize, araddr from the AR item passed via the slave vseq TLM FIFO.

- Reads arlen+1 data words from the memory model at the computed beat addresses.

- Asserts RLAST on the final beat; drives RVALID with configurable back-pressure delay per beat.

# 9. Protocol Modules

## 9.1 axi4_master_proto_module

- Contains: AW agent (ACTIVE), W agent (ACTIVE), AR agent (ACTIVE), B agent (PASSIVE), R agent (PASSIVE).

- B and R are passive on the master side — the master only receives responses, never drives them.

- Exposes five analysis ports forwarded from each agent monitor to the environment scoreboard and coverage.

- Holds a handle to the master virtual sequencer which the test starts via the vseq.

## 9.2 axi4_slave_proto_module

- Contains: AW agent (PASSIVE), W agent (PASSIVE), AR agent (PASSIVE), B agent (ACTIVE), R agent (ACTIVE).

- AW, W, AR are passive — the slave only receives requests, never drives address or write data channels.

- Exposes five analysis ports and a handle to the slave virtual sequencer.

# 10. Virtual Sequences

## 10.1 master_vseq

The master virtual sequence is the primary stimulus generator. It accepts the following knobs from the test, which propagate all the way through to the drivers and monitors:

| Knob | AXI4 field | Effect on drivers and monitors |
| --- | --- | --- |
| awsize / arsize | AWSIZE / ARSIZE | W driver computes WSTRB byte lanes per beat based on size and aligned offset; R driver packs RDATA into correct lanes; monitors reconstruct data per active lane |
| awlen / arlen | AWLEN / ARLEN | W driver loops exactly AWLEN+1 beats and asserts WLAST on the last beat; R driver drives exactly ARLEN+1 RDATA beats with RLAST on the final beat; monitors count beats and flag early or late WLAST/RLAST |
| awburst / arburst | AWBURST / ARBURST | Determines address increment rule applied per beat (FIXED, INCR, WRAP); monitors compute expected next address per burst type and compare against actual; WRAP boundary detected and flagged if crossed incorrectly |
| num_wr_txns | vseq iteration | Number of complete write transactions (AW phase then W phase then B response) generated per vseq run |
| num_rd_txns | vseq iteration | Number of complete read transactions (AR phase then R response with arlen+1 beats) generated per vseq run |

### AW → W ordering

The write path inside master_vseq always completes the AW handshake first, then triggers the W sequence with the exact awlen, awsize, and awburst from the completed AW item. The read path (AR) runs in a parallel fork so write and read transactions interleave naturally.

## 10.2 slave_vseq

- Monitors AW, W, and AR channels via TLM FIFOs fed by the passive slave-side monitors.

- Write path: on receiving WLAST, writes all beats into the memory model using WSTRB masking; then drives a B response with the matching BID.

- Read path: on receiving an AR item, reads arlen+1 data words from the memory model at the computed beat addresses; drives R beats with matching RID and RLAST on the final beat.

- Back-pressure (BVALID/RVALID delay) is configurable per transaction via resp_delay fields in axi4_env_cfg.

# 11. Memory Model  (axi4_mem_model)

A byte-addressed associative array shared between master and slave vseqs via the config DB. It serves as both the write target and the read reference model.

- write(addr, data, strb) — writes only the byte lanes indicated by WSTRB; unwritten lanes are unaffected.

- read(addr, len) — returns len+1 words starting from addr; addresses that were never written return 8'hx.

- The scoreboard uses the same memory model to predict expected RDATA for each beat and flags mismatches.

# 12. Scoreboard and Coverage

## 12.1 Scoreboard

- Subscribes to all ten analysis ports (five master-side, five slave-side monitors).

- Write path: AW items enter aw_queue[awid]; W bursts enter w_queue[awid]. On receiving B, scoreboard pops the matching entry and verifies BID and BRESP.

- Read path: AR items enter ar_queue[arid]. On receiving each R beat, scoreboard checks RID and rdata against the memory model; checks RLAST fires on beat arlen.

- ID-indexed queues handle out-of-order responses by ID without false errors.

## 12.2 Functional Coverage

- Burst type: bins for FIXED, INCR, WRAP on both AW and AR channels.

- Transfer size: bins for 1B, 2B, 4B, 8B (AWSIZE and ARSIZE).

- Burst length: bins for single-beat (len=0), short (1-15), medium (16-63), long (64-255).

- Transaction ID: all AWID and ARID values exercised.

- Response: OKAY, EXOKAY, SLVERR, DECERR bins for both BRESP and RRESP.

- Cross coverage: burst type x transfer size x burst length.

# 13. Test Modes

Test mode is selected via +TB_MODE=B2B or +TB_MODE=LOOPBACK passed to the simulator. The tb_top reads this plusarg and applies the correct interface wiring.

| Aspect | Back-to-back mode | Loopback mode |
| --- | --- | --- |
| Plusarg | +TB_MODE=B2B | +TB_MODE=LOOPBACK |
| Interface wiring | Master interface signals tied to slave interface signals in tb_top via assign statements | Master interface output ports connected back to same-side slave input ports |
| Master vseq | Runs normally; waits for slave vseq to respond with B/R | Drives both master and slave sequencers directly |
| Slave vseq | Monitors AW/W/AR via TLM FIFOs; drives B/R responses reactively | Same slave vseq; loopback resolves address routing internally |
| Scoreboard | Checks master-issued vs slave-received; matches B/R responses by ID | Source and sink are same protocol module; same scoreboard logic |
| Use case | Full VIP integration check; emulates real DUT connection | Single-module self-test; validates each protocol module independently |

# 14. Key Implementation Rules

- Never share seq items between channels. One class per channel eliminates constraint interference.

- All burst arithmetic — beat count, WSTRB, address increment — is computed from awlen/awsize/awburst in the seq item; never hardcoded in drivers or monitors.

- Drivers use clocking block outputs only. Monitors sample clocking block inputs only. No direct signal access outside clocking blocks.

- VALID must not be deasserted once asserted until READY is seen — enforced by both the protocol assertion in the interface and by driver logic.

- VALID must not depend on READY — drivers assert VALID unconditionally and then wait for READY; they never sample READY before asserting VALID.

- A write response (B) must always follow WLAST of its write transaction — slave vseq enforces this ordering via the TLM FIFO.

- Slave must not assert RVALID until both ARVALID and ARREADY have completed — enforced by the interface assertion and the slave vseq FIFO ordering.

- Reset handling: all drivers detect rst_n deassertion and flush any pending handshake cleanly before restarting.

- All widths, depths, and delays come from axi4_env_cfg via uvm_config_db — no magic numbers in any component.